BELT CONVEYOR CALCULATION REPORT

INPUT DATA

| Parameter | Value | Symbol | Unit |
| --- | --- | --- | --- |
| Required capacity | {{required_capacity}} | C_req | tph |
| Material | {{material_name}} | - | - |
| Bulk density | {{bulk_density}} | rho | t/m3 |
| Lump size type | {{lump_size_type}} | - | - |
| Lump size factor | {{lump_size_factor}} | F_l | - |
| Abrasiveness type | {{abrasiveness_type}} | - | - |
| Abrasiveness factor | {{abrasiveness_factor}} | F_a | - |
| Speed factor | {{speed_factor}} | SF_v | - |
| Belt speed | {{belt_speed}} | V | m/s |
| Initial material velocity | {{initial_velocity}} | V0 | m/s |
| Belt width | {{belt_width_mm}} | B | mm |
| Belt width | {{belt_width_m}} | B | m |
| Trough angle | {{trough_angle}} | lambda | degree |
| Surcharge angle | {{surcharge_angle}} | beta | degree |
| Cross-sectional area of load | {{cross_sectional_area}} | A | m2 |
| Conveyor length | {{conveyor_length}} | L | m |
| Lift / elevation | {{lift_elevation}} | H | m |
| Slope angle | {{slope_angle}} | delta | degree |
| Slope factor | {{slope_factor}} | K | - |
| Working time | {{working_time}} | t | hours |
| Service factor | {{service_factor}} | SF | - |
| Artificial friction coefficient | {{artificial_friction_coefficient}} | f | - |
| Mass of carrying idlers | {{mass_carrying_idlers}} | m_c | kg/m |
| Mass of return idlers | {{mass_return_idlers}} | m_r | kg/m |
| Mass of belt | {{mass_belt}} | m_B | kg/m |
| Coefficient of friction: material and belt | {{friction_material_belt}} | mu_1 | - |
| Coefficient of friction: material and skirt | {{friction_material_skirt}} | mu_2 | - |
| Skirt width | {{skirt_width}} | b_1 | m |
| Skirt length | {{skirt_length}} | l_sk | m |
| Belt thickness | {{belt_thickness}} | t_b | m |
| Pulley diameter | {{pulley_diameter}} | D | m |
| Shaft diameter at bearing | {{shaft_diameter_at_bearing}} | d | m |
| Vector sum of pulley tensions | {{vector_sum_tensions}} | R_v | N |
| Average belt tension | {{average_belt_tension}} | T_av | N/m |
| Trough factor | {{trough_factor}} | C_i | - |
| Friction between idler and belt | {{friction_idler_belt}} | mu_0 | - |
| Length with tilted idlers | {{length_tilted_idlers}} | L_i | m |
| Idler tilt angle | {{idler_tilt_angle}} | i | degree |
| Cleaner pressure | {{cleaner_pressure}} | p | N/m2 |
| Coefficient of friction: belt and cleaner | {{friction_belt_cleaner}} | mu_3 | - |
| Scraping factor | {{scraping_factor}} | K_s | N/m |
| Drive efficiency | {{drive_efficiency}} | eta_1 | - |
| Gravity | {{gravity}} | g | m/s2 |
| Drive coefficient | {{drive_coefficient}} | K_d | - |
| Wrap angle | {{wrap_angle}} | alpha | degree |
| External cleaner count | {{external_cleaner_count}} | n_e | - |
| Internal cleaner count | {{internal_cleaner_count}} | n_i | - |
| External cleaner thickness | {{external_cleaner_thickness}} | t_e | mm |
| Internal cleaner thickness | {{internal_cleaner_thickness}} | t_i | mm |
| External cleaner pressure | {{external_cleaner_pressure}} | p_e | N/m2 |
| Internal cleaner pressure | {{internal_cleaner_pressure}} | p_i | N/m2 |
| Pulley face width | {{pulley_face_width}} | L_p | mm |
| Allowable shear stress | {{allowable_shear_stress}} | tau | MPa |
| Motor speed | {{motor_speed}} | N_m | rpm |
| Selected motor rating | {{selected_motor_rating}} | P_sel | kW |

CALCULATIONS

1. Bulk density selection

rho = density corresponding to {{material_name}} from IS 8730 material-density table

= {{bulk_density}} t/m3

2. Speed factor

SF_v = F_l + F_a

= {{lump_size_factor}} + {{abrasiveness_factor}}

= {{speed_factor}}

3. Belt speed selection

V = recommended belt speed from belt-width and speed-factor table

= lookup({{belt_width_mm}}, {{speed_factor}})

= {{belt_speed}} m/s

4. Belt width conversion

B = B_mm / 1000

= {{belt_width_mm}} / 1000

= {{belt_width_m}} m

5. Cross-sectional area of load on belt

A = lookup from maximum-area table using belt width, surcharge angle and trough angle

= lookup({{belt_width_mm}}, {{surcharge_angle}}, {{trough_angle}})

= {{cross_sectional_area}} m2

6. Slope angle of conveyor

delta = tan^-1(H / L)

= tan^-1({{lift_elevation}} / {{conveyor_length}})

= {{slope_angle}} degree

7. Slope factor

K = lookup from slope-factor table using rounded inclination angle

= lookup(roundup_even({{slope_angle}}))

= {{slope_factor}}

8. Service factor

SF = 1.2 for working time < 8 h; 1.6 for 8-12 h; 2.0 for working time > 12 h

= service_factor({{working_time}})

= {{service_factor}}

9. Conveyor capacity

C = 3600 x rho x A x V x K

= 3600 x {{bulk_density}} x {{cross_sectional_area}} x {{belt_speed}} x {{slope_factor}}

= {{conveyor_capacity}} tph

10. Volumetric capacity

Q = C_req / (3600 x rho)

= {{required_capacity}} / (3600 x {{bulk_density}})

= {{volumetric_capacity}} m3/s

11. Mass of handled material per metre

m_G = 1000 x rho x Q / V

= 1000 x {{bulk_density}} x {{volumetric_capacity}} / {{belt_speed}}

= {{mass_handled_material}} kg/m

12. Main resistance

R = f x L x g x [m_c + m_r + (2 x m_B + m_G) x cos(delta)]

= {{artificial_friction_coefficient}} x {{conveyor_length}} x {{gravity}} x [{{mass_carrying_idlers}} + {{mass_return_idlers}} + (2 x {{mass_belt}} + {{mass_handled_material}}) x cos({{slope_angle}})]

= {{main_resistance}} N

13. Slope resistance

R_SL = m_G x H x g

= {{mass_handled_material}} x {{lift_elevation}} x {{gravity}}

= {{slope_resistance}} N

14. Acceleration length

l_a = (V^2 - V0^2) / (2 x g x mu_1), if V > V0; otherwise 0

= ({{belt_speed}}^2 - {{initial_velocity}}^2) / (2 x {{gravity}} x {{friction_material_belt}})

= {{accel_length}} m

15. Acceleration resistance due to inertia

R_a = 1000 x Q x rho x (V - V0)

= 1000 x {{volumetric_capacity}} x {{bulk_density}} x ({{belt_speed}} - {{initial_velocity}})

= {{accel_resistance_inertial}} N

16. Skirt resistance in acceleration zone

R_ska = [mu_2 x Q^2 x 1000 x rho x g x l_a] / [((V + V0)/2)^2 x b_1^2]

= [{{friction_material_skirt}} x {{volumetric_capacity}}^2 x 1000 x {{bulk_density}} x {{gravity}} x {{accel_length}}] / [(({{belt_speed}} + {{initial_velocity}})/2)^2 x {{skirt_width}}^2]

= {{skirt_accel_resistance}} N

17. Wrap resistance for non-drive pulley

R_w = 9 x B x [140 + 0.01 x (T_av / B)] x (t_b / D)

= 9 x {{belt_width_m}} x [140 + 0.01 x ({{average_belt_tension}} / {{belt_width_m}})] x ({{belt_thickness}} / {{pulley_diameter}})

= {{wrap_resistance}} N

18. Pulley bearing resistance

R_b = 0.005 x (d / D) x R_v

= 0.005 x ({{shaft_diameter_at_bearing}} / {{pulley_diameter}}) x {{vector_sum_tensions}}

= {{bearing_resistance}} N

19. Secondary resistance

R_s = R_a + R_ska + R_w + R_b

= {{accel_resistance_inertial}} + {{skirt_accel_resistance}} + {{wrap_resistance}} + {{bearing_resistance}}

= {{secondary_resistance}} N

20. Resistance due to idler tilting

R_i = g x C_i x mu_0 x L_i x (m_B + m_G) x cos(delta) x sin(i)

= {{gravity}} x {{trough_factor}} x {{friction_idler_belt}} x {{length_tilted_idlers}} x ({{mass_belt}} + {{mass_handled_material}}) x cos({{slope_angle}}) x sin({{idler_tilt_angle}})

= {{idler_tilt_resistance}} N

21. Main skirt-board resistance

R_sk = [mu_2 x Q^2 x 1000 x rho x g x l_sk] / [V^2 x b_1^2]

= [{{friction_material_skirt}} x {{volumetric_capacity}}^2 x 1000 x {{bulk_density}} x {{gravity}} x {{skirt_length}}] / [{{belt_speed}}^2 x {{skirt_width}}^2]

= {{skirt_resistance_main}} N

22. Belt cleaner resistance

A_1 = (n_e x B x t_e x p_e x mu_3) + (n_i x B x t_i x p_i x mu_3)

= ({{external_cleaner_count}} x {{belt_width_m}} x {{external_cleaner_thickness}} x {{external_cleaner_pressure}} x {{friction_belt_cleaner}}) + ({{internal_cleaner_count}} x {{belt_width_m}} x {{internal_cleaner_thickness}} x {{internal_cleaner_pressure}} x {{friction_belt_cleaner}})

R_bc = {{belt_cleaner_resistance}} N

23. Discharge plough resistance

R_p = B x K_s

= {{belt_width_m}} x {{scraping_factor}}

= {{discharge_plough_resistance}} N

24. Special resistance

R_sp = R_i + R_sk + R_bc + R_p

= {{idler_tilt_resistance}} + {{skirt_resistance_main}} + {{belt_cleaner_resistance}} + {{discharge_plough_resistance}}

= {{special_resistance}} N

25. Drive pulley speed

N_p = (60 x V) / (pi x D)

= (60 x {{belt_speed}}) / (pi x {{pulley_diameter}})

= {{drive_pulley_rpm}} rpm

26. Drive wrap resistance

R_wd = R_w for drive pulley, calculated using drive pulley diameter and belt data

= {{drive_wrap_resistance}} N

27. Drive bearing resistance

R_bd = R_b for drive pulley bearing

= {{drive_bearing_resistance}} N

28. Effective tension

T_E = R + R_SL + R_s + R_sp

= {{main_resistance}} + {{slope_resistance}} + {{secondary_resistance}} + {{special_resistance}}

= {{effective_tension}} N

29. Power at drive pulley

P_DP = (T_E x V) / 1000

= ({{effective_tension}} x {{belt_speed}}) / 1000

= {{power_at_drive_pulley}} kW

30. Absorbed power

P_A = P_DP + [(R_wd + R_bd) x V / 1000]

= {{power_at_drive_pulley}} + [({{drive_wrap_resistance}} + {{drive_bearing_resistance}}) x {{belt_speed}} / 1000]

= {{absorbed_power}} kW

31. Motor power

P_M = (P_A / eta_1) x 1.2

= ({{absorbed_power}} / {{drive_efficiency}}) x 1.2

= {{motor_power}} kW

32. Gearbox power

P_G = P_M x SF

= {{motor_power}} x {{service_factor}}

= {{gearbox_power}} kW

33. Belt tensions at drive pulley

T_1 = T_E x [(K_d / (e^(mu x alpha) - 1)) + 1]

= {{effective_tension}} x [({{drive_coefficient}} / (e^({{friction_material_belt}} x {{wrap_angle_radian}}) - 1)) + 1]

= {{tight_side_tension}} N

T_2 = T_1 - T_E = {{slack_side_tension}} N

34. High speed coupling torque

T_HS = (9550 x P_G) / N_m

= (9550 x {{gearbox_power}}) / {{motor_speed}}

= {{high_speed_coupling}} N-m

35. Low speed coupling torque

T_LS = (9550 x P_G) / N_p

= (9550 x {{gearbox_power}}) / {{drive_pulley_rpm}}

= {{low_speed_coupling}} N-m

36. Resultant pulley load

F = T_1 + T_2

= {{tight_side_tension}} + {{slack_side_tension}}

= {{resultant_pulley_load}} N

37. Maximum bending moment on pulley shaft

M = F x L_p / 4

= {{resultant_pulley_load}} x {{pulley_face_width}} / 4

= {{maximum_bending_moment}} N-mm

38. Equivalent torque

T_eq = sqrt[(2M)^2 + (1.5T_HS x 1000)^2]

= sqrt[(2 x {{maximum_bending_moment}})^2 + (1.5 x {{high_speed_coupling}} x 1000)^2]

= {{equivalent_torque}} N-mm

39. Pulley shaft diameter

d = [(16 x T_eq) / (pi x tau)]^(1/3)

= [(16 x {{equivalent_torque}}) / (pi x {{allowable_shear_stress}})]^(1/3)

= {{pulley_shaft_diameter}} mm

OUTPUT CALCULATIONS

| Parameter | Value | Symbol | Unit |
| --- | --- | --- | --- |
| Conveyor capacity | {{conveyor_capacity}} | C | tph |
| Volumetric capacity | {{volumetric_capacity}} | Q | m3/s |
| Mass of handled material | {{mass_handled_material}} | m_G | kg/m |
| Main resistance | {{main_resistance}} | R | N |
| Slope resistance | {{slope_resistance}} | R_SL | N |
| Acceleration length | {{accel_length}} | l_a | m |
| Acceleration resistance | {{accel_resistance_inertial}} | R_a | N |
| Skirt acceleration resistance | {{skirt_accel_resistance}} | R_ska | N |
| Wrap resistance | {{wrap_resistance}} | R_w | N |
| Bearing resistance | {{bearing_resistance}} | R_b | N |
| Secondary resistance | {{secondary_resistance}} | R_s | N |
| Idler tilt resistance | {{idler_tilt_resistance}} | R_i | N |
| Skirt resistance main | {{skirt_resistance_main}} | R_sk | N |
| Belt cleaner resistance | {{belt_cleaner_resistance}} | R_bc | N |
| Discharge plough resistance | {{discharge_plough_resistance}} | R_p | N |
| Special resistance | {{special_resistance}} | R_sp | N |
| Drive pulley speed | {{drive_pulley_rpm}} | N_p | rpm |
| Drive wrap resistance | {{drive_wrap_resistance}} | R_wd | N |
| Drive bearing resistance | {{drive_bearing_resistance}} | R_bd | N |
| Effective tension | {{effective_tension}} | T_E | N |
| Power at drive pulley | {{power_at_drive_pulley}} | P_DP | kW |
| Absorbed power | {{absorbed_power}} | P_A | kW |
| Motor power | {{motor_power}} | P_M | kW |
| Gearbox power | {{gearbox_power}} | P_G | kW |
| Tight side tension | {{tight_side_tension}} | T_1 | N |
| Slack side tension | {{slack_side_tension}} | T_2 | N |
| High speed coupling torque | {{high_speed_coupling}} | T_HS | N-m |
| Low speed coupling torque | {{low_speed_coupling}} | T_LS | N-m |
| Resultant pulley load | {{resultant_pulley_load}} | F | N |
| Maximum bending moment | {{maximum_bending_moment}} | M | N-mm |
| Equivalent torque | {{equivalent_torque}} | T_eq | N-mm |
| Pulley shaft diameter | {{pulley_shaft_diameter}} | d | mm |
| Selected motor rating | {{selected_motor_rating}} | P_sel | kW |