VIBRATING FEEDER CALCULATION REPORT

INPUT DATA

| Parameter | Value | Symbol | Unit |
| --- | --- | --- | --- |
| Material | {{material_name}} |  |  |
| Required capacity | {{required_capacity}} | C | tph |
| Largest lump size | {{largest_lump_size}} | Lm | mm |
| Large size material | {{large_size_material_percent}} |  | % |
| Length | {{trough_length}} | L | mm |
| Bed depth | {{bed_depth}} | D | mm |
| Slope | {{slope}} | θ | degree |
| Stroke | {{stroke}} | S | mm |
| Speed | {{speed}} | N | rpm |
| Weight of feeder | {{empty_vibrating_mass}} | M | kg |
| Number of motors | {{number_of_unbalance_motors}} | n |  |
| Number of springs | {{number_of_springs}} | s |  |

CALCULATIONS

Material Velocity, v = N × S × 0.08
 		           = {{design_material_velocity}} m/s
Width, B  = Q / (3600 × ρ × D × v)
  	      = {{required_capacity}} / (3600 × {{bulk_density}} × {{bed_depth}} × {{design_material_velocity}})
   	      = {{required_feeder_width}}
Material cross-sectional area, A = B × D = {{required_feeder_width}} × {{bed_depth}}
					         = {{material_cross_sectional_area}}
Material volume on feeder, V = A × L = {{material_cross_sectional_area}}  × {{trough_length}}
  					   = {{material_volume_on_feeder}}
Material mass on feeder, m    = V × ρ × 1000
				  = {{material_volume_on_feeder}} × {{bulk_density}} × 1000
				  = {{material_mass_on_feeder}}
Total vibrating mass used, M = {{total_vibrating_mass_used}}
Throw index                = {{throw_index}}
Throw classification = {{throw_classification}}

OUTPUT CALCULATIONS

| Parameter | Value | Symbol | Unit |
| --- | --- | --- | --- |
| Angular speed | {{angular_speed}} | Na | rad/s |
| Centrifugal force | {{centrifugal_force_total}} | F | kN |
| Motor power | {{motor_power_each_required}} | P | kW |
| Stiffness per spring | {{stiffness_per_spring}} | K | N/mm |