CONVEYOR DESIGN CALCULATION REPORT

# 1. Input Data

| Sl. No. | Input Parameter | Value | Unit |
| --- | --- | --- | --- |
| 1 | Rated capacity | {rated_capacity} | tph |
| 2 | Margin | {margin} | - |
| 3 | Design capacity | {design_capacity} | tph |
| 4 | Material handled | {material_name} | - |
| 5 | Bulk density | {bulk_density} | t/m³ |
| 6 | Lump Size | {lump_size} | - |
| 7 | Abrasiveness | {abrasiveness} | - |
| 8 | Belt speed | {belt_speed} | m/s |
| 9 | Conveyor length | {conveyor_length} | m |
| 10 | Lift | {lift} | m |
| 11 | Total skirt length | {total_skirt_length} | m |
| 12 | Belt width | {belt_width} | mm |
| 13 | Idler diameter | {idler_diameter} | mm |
| 14 | Belt type | {belt_type} | - |
| 15 | Carcass thickness | {carcass_thickness} | mm |
| 16 | Drive pulley diameter | {drive_pulley_diameter} | mm |
| 17 | Snub pulley diameter | {snub_pulley_diameter} | mm |
| 18 | Tail pulley diameter | {tail_pulley_diameter} | mm |

# 2. Calculation Details

## 2.1 Design Capacity

Qd = Qr × M

Qd = {rated_capacity} × {margin}

Qd = {design_capacity} TPH

## 2.2 Mass of Handled Material

mG = (Qd × 1000) / (3600 × v)

mG = ({design_capacity} × 1000) / (3600 × {belt_speed})

mG = {mass_handled_material} kg/m

## 2.3 Main Resistance

R = f × L × g × [mc + mr + (2mB + mG) × cos δ]

R = {artificial_friction_coefficient} × {conveyor_length} × {gravity} × [{mass_carrying_idlers} + {mass_return_idlers} + (2 × {mass_belt} + {mass_handled_material}) × {cos_slope_angle}]

R = {main_resistance} N

## 2.4 Slope Resistance

Rsl = mG × H × g

Rsl = {mass_handled_material} × {lift} × {gravity}

Rsl = {slope_resistance} N

## 2.5 Acceleration Length

la = effective acceleration length considered at loading zone

la = {acceleration_length} m

## 2.6 Acceleration Resistance

Ra = Q × 1000 × ρ × (v - v0)

Q = {volumetric_capacity} m³/s

Ra = {volumetric_capacity} × 1000 × {bulk_density} × ({belt_speed} - {initial_velocity})

Ra = {acceleration_resistance} N

## 2.7 Skirt Acceleration Resistance

Rska = (μ2 × Q² × 1000 × ρ × la × g) / [((v + v0)/2)² × b1²]

Rska = ({friction_material_skirt} × {volumetric_capacity}² × 1000 × {bulk_density} × {acceleration_length} × {gravity}) / [(({belt_speed} + {initial_velocity})/2)² × {skirt_width}²]

Rska = {skirt_acceleration_resistance} N

## 2.8 Wrap Resistance

Rw = nt × Rwt + ns × Rws

Rw = ({tight_side_pulley_count} × {tight_side_wrap_resistance}) + ({slack_side_pulley_count} × {slack_side_wrap_resistance})

Rw = {wrap_resistance} N

## 2.9 Bearing Resistance

Rb = nb × Rpb

Rb = {non_drive_pulley_count} × {bearing_resistance_per_pulley}

Rb = {bearing_resistance} N

## 2.10 Secondary Resistance

Rs = Ra + Rska + Rw + Rb

Rs = {acceleration_resistance} + {skirt_acceleration_resistance} + {wrap_resistance} + {bearing_resistance}

Rs = {secondary_resistance} N

## 2.11 Idler Tilt Resistance

Ri = g × Ci × μ0 × Li × (mB + mG) × cos δ × sin i

Ri = {gravity} × {trough_factor} × {friction_idler_belt} × {length_tilted_idlers} × ({mass_belt} + {mass_handled_material}) × {cos_slope_angle} × {sin_idler_tilt_angle}

Ri = {idler_tilt_resistance} N

## 2.12 Skirt Resistance Main

Rsk = (μ2 × Q² × 1000 × ρ × g × lsk) / (v² × b1²)

Rsk = ({friction_material_skirt} × {volumetric_capacity}² × 1000 × {bulk_density} × {gravity} × {total_skirt_length}) / ({belt_speed}² × {skirt_width}²)

Rsk = {skirt_resistance_main} N

## 2.13 Belt Cleaner Resistance

Rbc = A × P × μ3

Rbc = {belt_cleaner_area} × {cleaner_pressure} × {friction_belt_cleaner}

Rbc = {belt_cleaner_resistance} N

## 2.14 Discharge Plough Resistance

Rp = selected discharge plough resistance

Rp = {discharge_plough_resistance} N

## 2.15 Special Resistance

Rsp = Ri + Rsk + Rbc + Rp

Rsp = {idler_tilt_resistance} + {skirt_resistance_main} + {belt_cleaner_resistance} + {discharge_plough_resistance}

Rsp = {special_resistance} N

## 2.16 Drive Pulley Speed

np = (v × 60) / (π × Dd/1000)

np = ({belt_speed} × 60) / (π × {drive_pulley_diameter}/1000)

np = {drive_pulley_rpm} rpm

## 2.17 Drive Wrap Resistance

Rwd = resistance due to belt wrapping over the drive pulley

Rwd = {drive_wrap_resistance} N

## 2.18 Drive Bearing Resistance

Rbd = bearing resistance at drive pulley

Rbd = {drive_bearing_resistance} N

## 2.19 Effective Tension

Te = R + Rs + Rsp + Rsl

Te = {main_resistance} + {secondary_resistance} + {special_resistance} + {slope_resistance}

Te = {effective_tension} N

## 2.20 Power at Drive Pulley

Pdp = (Te × v) / 1000

Pdp = ({effective_tension} × {belt_speed}) / 1000

Pdp = {power_at_drive_pulley} kW

## 2.21 Absorbed Power

Pa = Pdp + ((Rwd + Rbd) × v / 1000)

Pa = {power_at_drive_pulley} + (({drive_wrap_resistance} + {drive_bearing_resistance}) × {belt_speed} / 1000)

Pa = {absorbed_power} kW

## 2.22 Motor Power

Pm = (Pa × k) / (η1 × η2)

Pm = ({absorbed_power} × {motor_factor}) / ({transmission_efficiency} × {derating_factor})

Pm = {motor_power} kW

## 2.23 Gearbox Power

Pg = service factor × Pm

Pg = {gearbox_service_factor} × {motor_power}

Pg = {gearbox_kw} kW

## 2.24 Tight Side Tension

T1 = Te × [(e / (e^(μθ) - 1)) + 1]

T1 = {effective_tension} × [({coupling_factor} / ({friction_drive_factor} - 1)) + 1]

T1 = {tight_side_tension} N

## 2.25 Slack Side Tension

T2 = Te × e × [1 / (e^(μθ) - 1)]

T2 = {effective_tension} × {coupling_factor} × [1 / ({friction_drive_factor} - 1)]

T2 = {slack_side_tension} N

## 2.26 High Speed Coupling Torque

THS = 100 × (Pr × 1.36 × service factor) / nm

THS = 100 × ({selected_motor_rating} × 1.36 × {coupling_service_factor}) / {motor_speed}

THS = {high_speed_coupling} kW/rpm

## 2.27 Low Speed Coupling Torque

TLS = Pr × service factor / no

no = {actual_output_rpm} rpm

TLS = {selected_motor_rating} × {coupling_service_factor} / {actual_output_rpm}

TLS = {low_speed_coupling} kW/rpm

## 2.28 Resultant Pulley Load

Fr = √[(T1 + T2 × cos α)² + (Wp - T2 × sin α)²] / 1000

Fr = √[({tight_side_tension} + {slack_side_tension} × {cos_alpha})² + ({drive_pulley_weight} - {slack_side_tension} × {sin_alpha})²] / 1000

Fr = {resultant_pulley_load} kN

## 2.29 Maximum Bending Moment

Mb = Fb × a

Mb = {bearing_load} × {bending_moment_arm}

Mb = {maximum_bending_moment} kN-cm

## 2.30 Equivalent Torque

Me = √[(2Mb)² + (1.5Mt)²]

Me = √[(2 × {maximum_bending_moment})² + (1.5 × {torsional_moment})²]

Me = {equivalent_torque} kN-cm

## 2.31 Pulley Shaft Diameter

d = [(Me × 16) / (π × τ)]^(1/3) × 10

d = [({equivalent_torque} × 16) / (π × {allowable_shear_stress})]^(1/3) × 10

dselected = {pulley_shaft_diameter} mm

# 3. Output Summary

| Sl. No. | Output Parameter | Value | Unit |
| --- | --- | --- | --- |
| 1 | Mass of handled material | {mass_handled_material} | kg/m |
| 2 | Main resistance | {main_resistance} | N |
| 3 | Slope resistance | {slope_resistance} | N |
| 4 | Acceleration length | {acceleration_length} | m |
| 5 | Acceleration resistance | {acceleration_resistance} | N |
| 6 | Skirt acceleration resistance | {skirt_acceleration_resistance} | N |
| 7 | Wrap resistance | {wrap_resistance} | N |
| 8 | Bearing resistance | {bearing_resistance} | N |
| 9 | Secondary resistance | {secondary_resistance} | N |
| 10 | Idler tilt resistance | {idler_tilt_resistance} | N |
| 11 | Skirt resistance main | {skirt_resistance_main} | N |
| 12 | Belt cleaner resistance | {belt_cleaner_resistance} | N |
| 13 | Discharge plough resistance | {discharge_plough_resistance} | N |
| 14 | Special resistance | {special_resistance} | N |
| 15 | Drive pulley speed | {drive_pulley_rpm} | rpm |
| 16 | Drive wrap resistance | {drive_wrap_resistance} | N |
| 17 | Drive bearing resistance | {drive_bearing_resistance} | N |
| 18 | Effective tension | {effective_tension} | N |
| 19 | Power at drive pulley | {power_at_drive_pulley} | kW |
| 20 | Absorbed power | {absorbed_power} | kW |
| 21 | Motor power | {motor_power} | kW |
| 22 | Gearbox power | {gearbox_kw} | kW |
| 23 | Tight side tension | {tight_side_tension} | N |
| 24 | Slack side tension | {slack_side_tension} | N |
| 25 | High speed coupling torque | {high_speed_coupling} | kW/rpm |
| 26 | Low speed coupling torque | {low_speed_coupling} | kW/rpm |
| 27 | Resultant pulley load | {resultant_pulley_load} | kN |
| 28 | Maximum bending moment | {maximum_bending_moment} | kN-cm |
| 29 | Equivalent torque | {equivalent_torque} | kN-cm |
| 30 | Pulley shaft diameter | {pulley_shaft_diameter} | mm |
| 31 | Selected motor rating | {selected_motor_rating} | kW |